Presentation of the introductory workshop to the

RPI Pico W

Gerardo Muñoz

gmunoz@udistrital.edu.co

UDFJC

Bogotá, Colombia

Feb. 2024

Raspberry Pico W

https://datasheets.raspberrypi.com/picow/pico-w-datasheet.pdf

Raspberry Pi Pico W has been designed to be a low cost yet flexible development platform for RP2040, with a 2.4GHz

wireless interface and the following key features:

• Dual-core cortex M0+ at up to 133MHz

• RP2040 microcontroller with 2MB of flash memory

• On-board single-band 2.4GHz wireless interfaces (802.11n, Bluetooth 5.2)

◦ Exposes 26 multi-function 3.3V general purpose I/O (GPIO)

◦ 23 GPIO are digital-only, with three also being ADC capable

◦12-bit 500ksps analogue to digital converter (ADC)

◦ 2 × UART, 2 × I2C, 2 × SPI, 16 × PWM channels

◦ 1 × timer with 4 alarms, 1 × real time clock

• 2 × programmable I/O (PIO) blocks, 8 state machines in total

◦ Flexible, user-programmable high-speed I/O

◦ Can emulate interfaces such as SD card and VGA

https://docs.micropython.org/en/latest/rp2/quickref.html

Operating conditions for the Pico W are largely a function of the operating conditions specified by its components.

- Operating Temp Max 70°C (including self-heating)

- Operating Temp Min -20°C

- VBUS 5V ± 10%.

- VSYS Min 1.8V

- VSYS Max 5.5V

Add the MicroPython firmware

https://projects.raspberrypi.org/en/projects/getting-started-with-the-pico/3

The onboard LED

https://projects.raspberrypi.org/en/projects/getting-started-with-the-pico/5

from machine import Pin

led = Pin(“LED”, Pin.OUT)

led(1)

from machine import Pin, Timer

led = Pin(“LED”, Pin.OUT)

timer = Timer()

def blink(timer):

led.toggle()

timer.init(freq=2.5, mode=Timer.PERIODIC, callback=blink)

Use digital inputs and outputs

https://projects.raspberrypi.org/en/projects/getting-started-with-the-pico/6

from machine import Pin

pin_a = Pin(16, Pin.IN, Pin.PULL_UP)

print(pin_a())

from machine import Pin

pin_a = Pin(16, Pin.IN, Pin.PULL_UP)

pin_b = Pin(17, Pin.IN, Pin.PULL_UP)

led = Pin("LED", Pin.OUT)

def callback(pin):

# led = a xor b

led(pin_a() ^ pin_b())

pin_a.irq( handler=callback)

pin_b.irq( handler=callback)

callback(None)

print("Ok")

from machine import Pin

import time

led = Pin(15, Pin.OUT)

button = Pin(14, Pin.IN, Pin.PULL_DOWN)

while True:

if button.value():

led.toggle()

time.sleep(0.5)

PIO (advanced, optional)

https://colab.research.google.com/github/GerardoMunoz/Curso_Python/blob/main/Comparaci%C3%B3n_while_IRQ_PIO.ipynb

File

import math

# Open a file for writing

file_name = "duty_cycle_data_sin.txt"

file = open(file_name, "w")

try:

# Define the number of data points

num_points = 100

# Generate duty cycle data for a sine waveform

for i in range(num_points):

angle = 2 * math.pi * i / num_points

duty_cycle = (math.sin(angle) + 1) * 50  # Scales sine values to 0-100%

file.write(f"{duty_cycle:.2f}\n")

print(f"Data written to {file_name}")

finally:

# Close the file

file.close()

PWM

from machine import Pin, PWM

# Define PWM pin

pwm_pin = Pin(18)

# Configure PWM

pwm = PWM(pwm_pin)

pwm.freq(1000)  # Set the PWM frequency (Hz)

duty_cycle = 10

pwm.duty_u16(int(65535 * duty_cycle / 100))

# Turn off PWM when done

#pwm.deinit()

from machine import Pin, PWM

import utime

# Define PWM pin

pwm_pin = Pin(18)

# Open the file for reading

file_name = "duty_cycle_data_sin.txt"

file = open(file_name, "r")

# Configure PWM

pwm = PWM(pwm_pin)

pwm.freq(1000)  # Set the PWM frequency (Hz)

try:

# Read duty cycles from the file

duty_cycles = [float(line.strip()) for line in file.readlines()]

finally:

# Close the file

file.close()

# Generate waveform based on duty cycles

for duty_cycle in duty_cycles:

pwm.duty_u16(int(65535 * duty_cycle / 100))

utime.sleep_ms(100)  # Adjust sleep time as needed

# Turn off PWM when done

pwm.deinit()

ADC

from machine import Pin, ADC

# Set up ADC on Pin 26

adc = ADC(0)

print(adc.read_u16())

from machine import Pin, Timer, ADC

import utime

# Set up ADC on Pin 26

adc = ADC(0)

# Open a file for writing

file_name = "adc_data_timer.txt"

file = open(file_name, "a") #a append the data

led = Pin("LED", Pin.OUT)

timer = Timer()

left_readings = 10

def blink(timer):

global left_readings

led.toggle()

adc_value = adc.read_u16()

file.write(str(adc_value) + "\n")

left_readings -= 1

if left_readings < 1:

timer.deinit()

file.close()

print("The END")

timer.init(freq=2.5, mode=Timer.PERIODIC, callback=blink)

ADC with DMA

https://github.com/GerardoMunoz/Curso_Python/blob/main/ADC_upy_DMA.ipynb